KESFLEX POLYURETHANE EXPANSION JOINT

DESCRIPTION

Kesflex Polyurethane Expansion Joint is a two part pourable grade polyurethane. Kesflex Polyurethane Expansion Joint is easy to mix, giving good self levelling properties and can be poured directly into horizontal joints.

USES

For sealing internal joints subject to heavy industrial use in factory and warehouse floors.

ADVANTAGES

Good chemical resistance to minerals, acids, alkalis and petroleum based fuels. (For specific chemical resistance date please contact our technical department.)

Provides a tough resilient cure Excellent adhesion, without a primer, to clean dry concrete.

Suitable for use in wide joints

Choice of colours

Good abrasion resistance

Good flexibility giving high movement capability

PROPERTIES

Pot Life (at 20C)	         	40 - 60 minutes

Service Temperature Range       -20C to 85C

Application Temperature Range  5Cto 30C

Initial Cure Time ( at 20C)         	24 hours (suitable for light traffic)

Full Cure (at 20C)	          	7 days

Shore D Hardness	          	60

S.G.			          	1.5

MIXING AND APPLICATION

Kesflex Polyurethane Expansion Joint system is supplied in correct mixing ratio. Add the full contents of the Curing Agent Component “B” into the Resin Component “A”. Mix together for at least five minutes until a uniform colour is obtained.

After a thorough mixing the material can be poured directly into the prepared joint to the required level. It maybe necessary after a few minutes to top up the level of the Kesflex Polyurethane Expansion Joint after it has flowed into all the void areas.

CLEANING

The uncured material and any tools can be cleaned off with a suitable solvent cleaner. Do not use for hand cleaning.

ESTIMATING

The total quantity of Kesflex Polyurethane Expansion Joint required can be worked out by using the following calculation factor:-

Joint Width (cm) x Jpint Depth (cm) x Joint Length (cm) / 1000 = No. Of Litres approx.

X 1.5 for kg.

PACKAGING

Kesflex Polyurethane Expansion Joint is available in 5KG units (approx 3.3L)

ADDITIONAL INFORMATION

Thixotropic grade can also be made available for areas where good sag resistance is required.

HEALTH AND SAFETY (see separate health and safety data sheet)

It is recommended that barrier creams, gloves and protective clothing is used when using Kesflex Polyurethane Expansion Joint. In case of contact with the eyes, rinse immediately with plenty of water and seek medical; advice. After contact with skin, wash immediately with plenty of soap and water.

WARRANTY DISCLAIMER

The technical data herein has been compiled for your help and guidance and is based upon our experience. As we have no control over the use to which our information is put, no warranty, expressed or implied is intended or given and we cannot be held responsible for coverage, performance or any damages incurred.

Kestral Construction Products Limited – St. Helens, WA9 4HY, 01744 830001: DATA S/781998